MONAGHAN TOWN COUNCIL

PLANNING AND DEVELOPMENT REPORT

File Ref: 13/801

This report has been prepared in accordance with Part 8 of the Planning and Development Regulations (S.I. No. 600 of 2001) and is being submitted to the members of Monaghan Town Council in the course of compliance with Section 179 of the Planning & Development Act 2000

Location: Mullaghcroghery, Killyconnigan, Tirkeenan & Tully

1.0   NATURE AND EXTENT OF THE PROPOSED DEVELOPMENT AND THE PRINCIPAL FEATURES THEREOF

  - The proposed development will involve the development of the Ulster Canal Greenway – Phase 1 scheme for approximately 3.4km in length. The proposed development works are to be located adjacent to the towpath of the Ulster Canal (with sections alongside the R189, Rooskey Vale, Old Cross Square, Canal Street, Glen Road, St. Louis Convent Grounds, Canal View Area, Oriel Way, Tom Young’s Wood and Killyconnigan Housing Area).

  - The proposed development works are to be located at Tirkeenan, Tully, Mullaghcroghery and Killyconnigan.

  - The proposed scheme will comprise of the following:

• Construction of 2730m of 3.5m wide bitmac path for cycling and walking provision

• Installation 6m lighting columns at 25m centres over a distance of 2220m.

• Construction of a 2.5m wide bike lane on Rooskey Vale. (Length 170m)

• Construction of a 1.5m wide contra-flow bike lane on Canal Street. (Length 200m)

• Upgrade of the existing pedestrian crossing on Glen Road to a Toucan crossing

• Construction of a new Toucan crossing on the Dublin Road side of Old Cross Square

• Installation of traffic calming measures on Park Road, Oriel Way, Killyconnigan and Glen Road.

• Widening of 110m of footpaths on Glen Road.

• Widening of 50m of footpaths on Oriel Way.

• Erection of approx. 915m of 2.4m high Paladin fencing

• Erection of approx. 980m of 1.2m high timber post and rail fencing.

• Widening of existing towpath under stone arch bridge at Killyconnigan

• Installation of 2No. CCTV Cameras at Horseshoe Bridge and Canal View.

• Construction of Bridge at Tully across the Ulster Canal to facilitate pedestrian and cycle movements.

• Removal of 18 no. long term parking spaces at Rooskey and 14 short term parking spaces on Canal Street.

2.0  SITE DESCRIPTION

2.1  	The site area comprises lands adjacent to and including portions of the towpath associated with a section of the Ulster Canal through Monaghan Town and its related environs.

3.0  CONSISTENCY WITH THE PROPER PLANNING AND SUSTAINABLE DEVELOPMENT OF THE AREA.

3.1 	The development is consistent with objectives and policies as contained within the Monaghan County Development Plan 2013-2019, as stated below;

Objectives

GIO1  Develop a Green Infrastructure Strategy within County Monaghan during the life time of the Development Plan, ensuring the protection of County Monaghan’s Natura 2000 sites is central to this strategy.

GIO2  Seek a net gain in green infrastructure through the protection and enhancement of existing assets, through the provision of new green infrastructure where appropriate, and in consultation with stakeholders, as an integral part of the planning process.

CWO2  Support, promote and facilitate walking and cycling as alternative modes of transport in appropriate locations throughout the County.

CWO4  Support, promote and encourage the development and maintenance of looped walks (including long and medium distance trails) throughout the County in line with the Government’s Smarter Travel Policy, particularly those which have cultural or historic association, which provides linkages with trails to existing established national, local and cross border walking/cycling routes.

CWO6  Identify safe and convenient walking and cycling routes in urban areas and between the main towns and villages in the County, and provide signage and ancillary facilities at appropriate locations to promote their use, during the lifetime of the plan.

TRO4  Promote high quality, flexible and responsive local transport services in urban and rural communities.

TRO5  Promote and facilitate the use of cycling and walking as alternative sustainable modes of transport in accordance with the provisions of the National Cycle Framework 2009-2020.

4.0   ASSESSMENT OF PROPOSED WORKS

Visual impact

4.1  	The site area is low-lying relative to the surrounding area and there are consequently no issues in relation to the proposed development resulting in adverse visual impact in the wider area.  Visual impact of any embankments or re-grading works associated with the development will be limited to views from the immediate locality and the proposed Greenway.

This is considered acceptable in terms of visual impact.

Archaeological and Architectural Heritage

4.2  	The stone arch bridge at Annahagh Road is a protected structure and the proposed Greenway is to pass underneath this bridge.  However, there are no works proposed which will affect this bridge.  On this basis there no issues with the proposed development affecting this protected structure.

There are no features of archaeological interest along the route of the proposed works.

Ecological impact

4.3    An ecological survey has been carried out as part of this development proposal by appointed environmental consultants.  Detailed surveys were carried out in respect of vegetation and survey work was carried out to determine if any protected species are located along the route of the proposed works.

As detailed in the survey, potential locations for bat roosts were found and it is recommended that a survey of the site area be carried out prior to commencement of works, in order to facilitate any mitigation measures if necessary.

Furthermore, potential locations for badger setts were noted at various lengths of the proposed Greenway given the existing vegetation and landform at various locations.  Again, it is recommended that a survey of the site area be carried out prior to commencement of works, in order to facilitate any mitigation measures if necessary.

In respect of existing vegetation along the proposed route, the ecological survey as carried out did not find any vegetation or trees of note and there are not considered to be any issues in this regard.

Impacts of Earthworks

4.4	There will be a limited degree of site levelling and re-grading works are included in the development proposal. Any surplus material to be removed off-site is to be deposited at licensed and permitted sites.

Impacts on existing footpaths

4.5  From the northern most point of the proposed works, the first connection of the Greenway with existing footpaths within Monaghan Town will be as the Greenway passes through Rooskey Vale and then passing through Old Cross Square.  The Greenway will be constructed across the parking spaces which are to be lost at this location.

Extending from Old Cross Square the Greenway will continue along Canal Street and then connecting with the existing walkway through the Ulster Canal Park.

Existing footpaths along the Glen Road are to be widened to 3 metres with provision of a Toucan pedestrian crossing, from where users of the greenway will have to cross the Glen Road.

Further along the route, as it extends south-westwards, through Mullaghmatt, Oriel Way and Killyconnigan existing footpaths are to be widened to a width of 3 metres until the Greenway then rejoins the towpath associated with the Ulster Canal.

Impacts on car parking

4.6    The proposed works will result in the loss of 18no. car parking spaces between Rooskey Vale and Old Cross Square and a further 14no. car parking spaces at Canal Street.

Notwithstanding the loss of these car parking spaces, a usage survey carried out by Monaghan Town Council has shown the applicable spaces are rarely used and thus it is considered that the loss of the car parking spaces will not cause any disruption.

Ancillary works

4.7  Included in the development proposal are works relating to provision of the following features:

• Erection of approx. 915m of 2.4m high Paladin fencing

• Erection of approx. 980m of 1.2m high timber post and rail fencing.

• Widening of existing Towpath under Stone Arch Bridge at Killyconnigan

• Installation of 2No. CCTV Cameras at Horseshoe Bridge and Canal View.

• Construction of a Bridge at Tully across the Ulster Canal to facilitate Pedestrian and Cycle movements.

The provision of the works as detailed above are considered to be acceptable.

Appropriate Assessment

4.8 	Under Article 6(3) of the EU Habitats Directive and Regulation 30 of SI no. 94/1997 “European Communities (Natural Habitats) Regulations” (1997) any plan or project which has the potential to significantly impact of the integrity of a Natura 2000 site (ie. SAC or 	SPA) must be subject to an Appropriate Assessment.

In accordance with this requirement, a screening report has been prepared to accompany this development proposal and it has been determined that there will be no significant impact on the integrity of any Natura 2000 sites and consequently an Appropriate Assessment 	is not required.

# 5.0	 INIDIVIDUAL SUBMISSIONS

5.1  A submission has been received from Ms. Maura O’Connell (c/o George Wright Solicitors).  This submission raises the following points:  (in summary)

- The proposed Greenway may encroach onto the lands as owned by Ms. O’Connell;

- The proposed Greenway may give rise to issues of noise, disruption, litter and antisocial behaviour;

- The proposed Greenway will detract from the privacy of the lands;

- The proposed Greenway will give rise to increased pedestrian activity.

Consideration

The respective points as submitted on behalf of Ms. O’Connell are noted.  In response the following points are highlighted:

- The proposed Greenway does not encroach onto the lands as owned by Ms. O’Connell.  The Town Council has carried out land-ownership searches of the applicable site area required for the Greenway project and is satisfied that lands owned by Ms. O’Connell are not affected by the proposed project.

- The design of the Greenway project has been designed so as to incorporate street lighting throughout with CCTV provision at 2no. separate locations so as to effectively manage any potential issues of anti-social behaviour.

- The lands within the ownership of Ms. O’Connell do not encompass a residential property (or extant permission) and is used for grazing of animals.  Accordingly it is not considered that the proposed Greenway will detract from the privacy of the lands.

5.2  A submission has also been received from Mr. Paudge Gormley.    This respective submission raises the following points:  (in summary)

- Concerns raised that the route of the proposed greenway is removed from the Ulster Canal at places;

- Concerns raised regarding the crossing points at the Horseshoe Bridge and also at Oriel Way;

- Highlights that the proposed Greenway is not optimizing the qualities of the Ulster Canal given the separation distance at intervals between the Greenway and the Ulster Canal; in particular he raises concerns regarding the sections which passes through Mullaghmatt and Cortolvin;

- Concerns regarding the surface finish of the proposed Greenway;

- Concerns raised regarding the potential for anti-social behaviour; requests CCTV cameras and palisade fencing in the vicinity of his dwelling to ensure his security and residential amenity is not affected by the proposal.

Consideration

The respective points as submitted from Mr Gormley are noted.  In response the following points are highlighted:  (attached overleaf)

- The site layout and location of the proposed Greenway has been designed to be as close as practical to the Ulster Canal for the entirety of the route.  Furthermore it is highlighted that the Ulster Canal and the associated towpath as existing has been encroached upon by three properties in the past and the design of the Greenway as proposed is so as to avoid major disruption to these properties.

- Pedestrian crossing points have been designed in accordance with best practice so as to ensure safety of pedestrian, cycle and vehicular traffic is facilitated throughout the length of the proposed Greenway.  Furthermore, it is highlighted that the entire project has been subject to a Stage 1 Road Safety audit which was part of the overall initial design for the Greenway and the applicable section as referred to by Mr Gormley was highlighted and addressed in the Audit. The project is also subject to a stage 2 Road Safety Audit for the Detailed Design.  The Road safety issues cited by Mr Gormley have been addressed by traffic calming and staggered gated entrances onto the Greenway and the table top crossings will provide a safe crossing for pedestrians and cyclists.

- The Greenway is to be finished to the specification as per scheme drawings, subject to funding.

- The Greenway project has been designed so as to incorporate street lighting throughout with CCTV provision at 2no. separate locations so as to effectively manage any potential issues of anti-social behaviour.  Furthermore, the provision of pallidan fencing for a length of 915 metres along the route where the Greenway is in proximity to residential dwellings will ensure the amenity of residents in the vicinity of the proposed project is protected where practicable.

7.0	EXTERNAL CONSULTATIONS/SUBMISSIONS

An Taisce:  		      No comments received

Inland Fisheries Ireland:  No specific objections

Waterways Ireland:  	      No objections

Failte Ireland:                  No comments received

Department of Arts Heritage and Gaelteacht:  No objections

National Roads Authority:  No objections

8.0  INTERNAL CONSULTATIONS/SUBMISSIONS

Water Services:  No objections;

Town Engineer:  No objections;

Environment Section: No objections;

9.0 RECOMMENDATION – Having considered the proper planning and sustainable development of the area it is recommended that Monaghan Town Council proceed with the development as proposed in accordance with the plans and documentation submitted on the 4th April 2013 and the conditions attached below:

1a. All works relating to watercourses affected by the development shall be carried out in consultation with Inland Fisheries Ireland. Relevant requirements of the document as published by Eastern Regional Fisheries Board (ERFB) entitled “Requirements for the protection of Fisheries Habitat during Construction and Development Works at River Sites” shall be incorporated into the works, unless alternative equivalent measures are agreed by the ERFB.

Reason:  To protect the aquatic environment

2a.	Prior to commencement of development, or as otherwise agreed in writing with the Planning Authority, details of any proposed off-site disposal of excavated soil or other construction waste shall be submitted to the Planning Authority for agreement in writing. The submitted information shall include details of the location of proposed disposal sites.

b.	During construction stage all efforts to be made to prevent the discharge of silt and suspended solids to any watercourse or wetland area.

c.	Development works shall be carried out in accordance with details as agreed with the Planning Authority under part (a) of this condition.

Reason:  In the interest of environmental protection.

3a. Prior to commencement of works, details shall be agreed with Water Services Section of Monaghan County Council in relation to water main pipelines and all associated matters in respect of Water Services affected by the proposed development.

b. The development shall be carried out in accordance with the agreed details.

Reason:  To secure a satisfactory standard of development.

4a. Details of a Project Management Plan and a Construction and Demolition Waste Management

Plan shall be submitted, for agreement, to the Planning Authority before any part of the

development is commenced.   Development shall be carried out in full accordance with such

details as have been agreed.

b) The Project Management Plan shall include the following measures

- Mitigation measures to be employed in respect of environmental protection (inclusive of natural heritage) both during construction and operational phases of the development as hereby approved.

- Provision for the temporary off-street accommodation of building materials, vehicles and temporary offices

(iii)        Car and truck parking facilities to be provided on-site during the Construction Phases of  development.

(iv)        Provision of a defined Contractor’s Compound incorporating measures, where necessary,  to counteract ground-water pollution.

(v)       A temporary wheel wash arrangement to be used by all vehicles exiting the site.

Reason:  In the interest of orderly development and environmental protection.

5a. Prior to commencement of development, or as otherwise agreed in writing with the Planning Authority, a survey shall be carried out by an appropriately qualified ecologist of the site area for the presence of bats and/or bat roosts.

b.   In the event that the presence of bats or roosts is detected by the survey as required under part (a) above, the applicant shall agree details in writing with the Planning Authority for the mitigation of impacts of the proposed development on any bats or related roosts within the site area.

c.   All details in respect of the site survey as required under part (a) of this condition and mitigation measures in respect of any bats or related roosts as may be required under part (b) of this condition shall be submitted to and agreed in writing with the Planning Authority prior to any construction works being carried out on the development.

d.  Development works shall be carried out in accordance with details as agreed with the Planning Authority under part (b) of this condition.

Reason:  To ensure the development does not impact on any bats or their related roosts and provide for protection of bats, if necessary.

6a.  Prior to commencement of development, or as otherwise agreed in writing with the Planning Authority, a survey shall be carried out by an appropriately qualified ecologist of the site area for the presence of badgers and/or badger setts.

b.   In the event that the presence of badgers or badger setts are detected by the survey as required under part (a) above, the applicant shall agree details in writing with the Planning Authority for the mitigation of impacts of the proposed development on any badgers or related setts within the site area.

c.   All details in respect of the site survey as required under part (a) of this condition and mitigation measures in respect of any badgers or related setts as may be required under part (b) of this condition shall be submitted to and agreed in writing with the Planning Authority prior to any construction works being carried out on the development.

d.  Development works shall be carried out in accordance with details as agreed with the Planning Authority under part (b) of this condition.

Reason:  To ensure the development does not impact on any badgers or their related setts and provide for protection of badgers, if necessary.

7.  The works shall be carried out in accordance with details as submitted to the Planning Authority on the 4th April 2013, except as may be necessary in order to comply with the above conditions.

Reason:  To secure a satisfactory standard of development.

________________________				__________________

Date

David Fallon

County Manager